Table S1. Recommended DW-MRI acquisition parameters for cell pellet phantoms

| Parameter | In this protocol | Notes |
| --- | --- | --- |
| Pulse sequence | Stimulated echo (STE) | Bruker DtiStandard sequence. A stimulated echo is required to achieve the long diffusion interval (Δ = 300 ms) needed for aquaporin contrast |
| Scanner | Bruker 7 T vertical-bore (154 mm bore) | 66 mm diameter rf coil (MiniSWB90) |
| Nominal b-values (s/mm2) | 0, 400, 600, 800 | As set in the sequence. Must use effective b-values for ADC fitting. Effective values include contributions from imaging and spoiler gradients and are extracted from the scanner method file. |
| Diffusion gradient duration (δ) | 5 ms |  |
| Diffusion interval (Δ) | 300 ms | This extended interval is critical–it allows water molecules to undergo multiple interactions with the plasma membrane |
| Diffusion gradient direction | 1 (readout direction, dorsal-ventral axis) | Single direction applied orthogonal to the axial slice selection gradient |
| Echo time (TE) | 19.992 ms |  |
| Repetition time (TR) | 1,000 ms |  |
| Field of view (FOV) | 42 × 42 mm |  |
| In-plane resolution | 0.328 × 0.328 mm2 |  |
| Slice thickness | 1–2 mm |  |
| Number of averages | 5 |  |

Dataset S1. Mean grey values obtained using ImageJ for different ROIs corresponding to four effective b-values

| Eff b\ROI | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| 1115.492573 | 27698.525 | 26243.896 | 27214.164 | 14535 | 15647.145 | 19359.805 |
| 1793.837271 | 23152.896 | 21407.805 | 22269.516 | 9815.36 | 10909.154 | 14458.289 |
| 2293.837271 | 20237.65 | 18864 | 19509.846 | 7078.134 | 8428.412 | 10861.278 |
| 2793.837271 | 17987.164 | 17426.805 | 17435.969 | 5923.794 | 6993.052 | 8374.34 |
|  | 56-dox | 56-dox | 56-dox | 56+dox | 56+dox | 56+dox |
| Calculated ADC | 0.260 | 0.255 | 0.271 | 0.567 | 0.500 | 0.490 |